Course Code : ECT 451-2 GVHW/RW – 23 / 1025

Seventh Semester B. Tech. ( Electronics and Communication Engineering ) Examination

MICROWAVE THEORY AND TECHNIQUES

Time : 3 Hours] [Max. Marks : 60

Instructions to Candidates :—

(1) All questions are compulsory.

(2) All questions carry marks as indicated against them.

(3) Due credit will be given to neatness and adequate dimensions. (4) Assume suitable data and illustrate answers with neat sketches wherever necessary.

1. (a) Outline a short history of Microwave Engineering Development. 5(CO1)

(b) Translate the First Five IEEE Microwave Bands in terms of its wavelength. 5(CO1)

2. (a) Summarize Planar type Transmission Lines. 6(CO2)

(b) In a unmatched transmission line, the total power incident on Port 1 is 5 W, whereas 2.2 W is reflected back. Power delivered to the load is 2 W at Port 2. Solve for (in dB) :

(i) Attenuation Loss.

(ii) Reflection Loss.

(iii) Insertion Loss. 4(CO2)

3. (a) Recall application of Faraday Rotation in Ferrites using the working of Gyrator. 5(CO4)

(b) A signal of 32 W is fed into one of the collinear ports of a loss-less H-Plane Tee junction. Discover the powers in the remaining ports when others are terminated by means of matched load. 5(CO4)

GVHW/RW-23 / 1025 Contd.

4. (a) A Circular MSA is to be designed at 2.45 GHz on a substrate having εr = 2.55, h = 1.66 mm, tanδ = 0.0012. Predict the radius of a circular antenna and feed point location for 50 Ω impedance matching. 6(CO3)

(b) Outline Periodic Structures in transmission lines. 4(CO3)

5. (a) Illustrate Antenna Gain measurement setup in anechoic chamber with block diagram. 6(CO5)

(b) Two identical 30 dB directional coupler are used to sample incident and reflected power in a waveguide with VSWR = 2. The output of the coupler sampling incident power is 4.5 mW. Estimate the value of reflected power. 4(CO5)

6. (a) Distinguish between Continuous Wave Doppler and Pulsed RADAR. 5(CO1)

(b) Estimate the range of a RADAR system which operates at 3 cm with a peak pulse power of 600 KW, if its antenna is 5 m2, minimum detectable signal is 10–13 W and the cross sectional area of the target is 20 m2. 5(CO1)

D

GVHW/RW-23 / 1025 2 85